SYSTEM / HARDWARE REQUIREMENTS SPECIFICATION

BCH(15,7,2) Error Detection and Correction Engine

| Project | BCH(15,7,2) Error Detection and Correction Engine |
| --- | --- |
| Document | System / Hardware Requirements Specification |
| Version | 1.0 |
| Date | 10 September 2026 |
| Author | Student |
| Target Platform | Basys 3 / Artix-7 FPGA |
| Target Device | xc7a35tcpg236-1 |
| HDL / Tool | SystemVerilog / Vivado 2021.1 |

# Document Control

| Version | Date | Author | Reviewer | Description |
| --- | --- | --- | --- | --- |
| 1.0 | 10-Sep-2026 | Student | — | Initial BCH project requirements |

## Document Status

☐ Draft    ☐ Under Review    ☐ Approved    ☐ Obsolete

## Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Student | Student | — | 10-Sep-2026 |

# 1. Purpose

Define what the processor must do and establish the approved baseline against which architecture, RTL, verification and implementation are measured.

# 2. Product / System Overview

The system is a BCH(15,7,2) error detection and correction engine implemented in SystemVerilog for an Artix-7 FPGA. It accepts 7-bit input data, generates a 15-bit BCH codeword containing 7 data bits and 8 parity bits, provides controlled injection of zero, one, or two bit errors, and uses syndrome-based decoding to detect and correct up to two erroneous bits. The corrected 7-bit data and corrected 15-bit codeword are produced as outputs. The design demonstrates reliable digital communication and reliable computing using forward error correction.

# 3. Scope

In scope: BCH(15,7,2) encoding, zero/one/two-bit error injection, syndrome calculation, GF(16) arithmetic, single- and double-bit error correction, error-status indication, RTL simulation, FPGA synthesis and implementation, Basys 3 pin constraints, timing analysis, power estimation, and bitstream generation.

Out of scope / future: correction of three or more bit errors, physical FPGA-board validation without a board, ASIC physical design, and higher-order BCH codes.

Out of scope / future: caches, branch prediction, CSR/interrupts, MMU/TLB, multicore.

# 4. Assumptions and Constraints

Target FPGA/device: Basys 3-compatible Artix-7, xc7a35tcpg236-1.
Target timing constraint: 15 ns virtual-clock period (66.67 MHz requirement used for the implemented combinational design).
Clock/reset assumptions: The BCH datapath is combinational and has no clock or reset ports.
Memory assumptions: No external memory is required; all processing is performed combinationally.

Target frequency: __________ MHz

Clock/reset assumptions: __________________

Memory assumptions: __________________

# 5. Functional Requirements

Each requirement is uniquely identified by a REQ-ID and is measurable and testable.

| REQ-ID | Category | Requirement | Priority | Verification Method |
| --- | --- | --- | --- | --- |
| REQ-01 | Functional | Accept 7-bit data and produce a 15-bit BCH codeword. | High | RTL simulation |
| REQ-02 | Encoding | Generate BCH parity using g(x)=x^8+x^7+x^6+x^4+1. | High | RTL simulation |
| REQ-03 | Error Injection | Support zero, one and two-bit error injection. | High | RTL simulation |
| REQ-04 | Decoding | Calculate BCH syndromes using GF(16) arithmetic. | High | RTL simulation / review |
| REQ-05 | Correction | Correct supported single-bit errors. | High | Verification tests |
| REQ-06 | Correction | Correct supported two-bit errors. | High | Verification tests |
| REQ-07 | Status | Report 00=no error, 01=one-bit, 10=two-bit, 11=uncorrectable. | High | Simulation |
| REQ-08 | Output | Provide corrected 7-bit data and 15-bit codeword. | High | Simulation |
| REQ-09 | FPGA | Map top-level I/O using Basys 3 XDC constraints. | Medium | Implementation |
| REQ-10 | Timing | Meet the specified 15 ns implementation timing requirement. | High | Static timing analysis |
| REQ-11 | Implementation | Complete synthesis, implementation and bitstream generation. | High | Vivado reports |

# 6. External Interfaces

| Interface | Direction | Width | Description | Timing/Protocol |
| --- | --- | --- | --- | --- |
| data_in | Input | 7 | Original user data. | Combinational |
| error_type | Input | 2 | 00=no error, 01=single-bit, 10=two-bit. | Combinational |
| error_pos1 | Input | 4 | First error bit position, 0–14. | Combinational |
| error_pos2 | Input | 4 | Second error bit position, 0–14. | Combinational |
| encoded_codeword | Output | 15 | BCH encoded codeword. | Combinational |
| corrupted_codeword | Output | 15 | Codeword after selected error injection. | Combinational |
| corrected_codeword | Output | 15 | Decoder-corrected codeword. | Combinational |
| corrected_data | Output | 7 | Recovered original data. | Combinational |
| error_status | Output | 2 | Correction status code. | Combinational |

The top-level interface consists of 7 data inputs, 2 error-type inputs, 4-bit position inputs for each possible error, and outputs for encoded, corrupted and corrected codewords, corrected data, and error status.

# 7. Performance Requirements

| Metric | Target | Measurement Method | Acceptance Criteria |
| --- | --- | --- | --- |
| Timing requirement | 15 ns period | Vivado implemented timing report | WNS >= 0 |
| Worst slack | +1.286 ns | Vivado Timing Summary | Pass |
| LUT utilization | 65 LUTs (0.31%) | Vivado utilization report | Pass |
| Power estimate | 0.12 W, low confidence | Vivado power report | Documented |
| Bitstream | Successfully generated | Vivado Generate Bitstream | Pass |

The implementation shall meet the specified 15 ns timing requirement, correctly process the supported zero/one/two-bit error cases, and remain within the available Artix-7 FPGA resources.

# 8. Error / Exceptional Conditions

| Condition | Required Behavior | Detection | Verification |
| --- | --- | --- | --- |
| No error | Pass received codeword unchanged | Zero syndromes | No-error simulation |
| One-bit error | Locate and correct one erroneous bit | Syndrome relationship | Single-bit tests |
| Two-bit error | Locate and correct two erroneous bits | BCH syndrome equations | Two-bit tests |
| 3+ errors | Not guaranteed to correct | Uncorrectable condition | Design limitation |

Zero errors shall pass through unchanged. One-bit errors shall be corrected and reported with status 01. Two-bit errors shall be corrected and reported with status 10. Unsupported or uncorrectable conditions shall be indicated with status 11. The error injector intentionally supports only zero, one and two-bit errors.

# 9. Acceptance Criteria

| Acceptance ID | Criterion | Pass Condition | Evidence |
| --- | --- | --- | --- |
| ACC-01 | BCH encoding operates correctly | Expected 15-bit codeword produced | Simulation waveform |
| ACC-02 | Single-bit correction works | Corrected codeword equals encoded codeword | Single-bit waveform |
| ACC-03 | Two-bit correction works | Corrected codeword equals encoded codeword | Two-bit waveform |
| ACC-04 | Status is correct | Status matches error condition | Simulation |
| ACC-05 | Implementation completes | Synthesis and implementation complete | Vivado reports |
| ACC-06 | Timing is met | WNS is non-negative | Timing Summary |
| ACC-07 | Bitstream generated | Generate Bitstream completes | Vivado status |

The design is accepted when RTL simulation demonstrates correct encoding and correction for supported cases, synthesis and implementation complete successfully, specified timing constraints are met, and a valid FPGA bitstream is generated.

# 10. Traceability

| Requirement | Architecture Item | RTL Item | Test Case | Result |
| --- | --- | --- | --- | --- |
| REQ-01 | BCH encoder | bch_encoder.sv | TB-01 | Verified |
| REQ-02 | BCH encoder | bch_encoder.sv | TB-01 | Verified |
| REQ-03 | Error injector | error_injector.sv | TB-02/TB-03 | Verified |
| REQ-04 | BCH decoder | bch_decoder.sv | TB-02/TB-03 | Verified |
| REQ-05 | BCH decoder | bch_decoder.sv | TB-02 | Verified |
| REQ-06 | BCH decoder | bch_decoder.sv | TB-03 | Verified |
| REQ-07 | Top/decoder | bch_top.sv / bch_decoder.sv | TB-01–TB-03 | Verified |
| REQ-08 | Top/decoder | bch_top.sv / bch_decoder.sv | TB-01–TB-03 | Verified |
| REQ-09 | FPGA constraints | bch_basys3.xdc | IMP-01 | Verified |
| REQ-10 | Timing constraint | bch_timing.xdc | STA-01 | Verified |
| REQ-11 | FPGA implementation | Vivado project | IMP-01 | Verified |

Requirements are traced to the BCH encoder, error injector, syndrome-based decoder, top-level integration, simulation testbench, XDC constraints, synthesis/implementation reports, timing report, power report, and generated bitstream.