ARCHITECTURE SPECIFICATION

BCH(15,7,2) Error Detection and Correction Engine

| Document Field | Value |
| --- | --- |
| Project | BCH(15,7,2) Error Detection and Correction Engine |
| Document | Architecture Specification |
| Version | 1.0 |
| Date | 10 September 2026 |
| Author | Student |
| Target Platform | Basys 3 / Artix-7 FPGA |
| Target Device | xc7a35tcpg236-1 |
| HDL / Tool | SystemVerilog / Vivado 2021.1 |
| Design Type | Combinational BCH error-correction engine |
| Correction Capability | Up to two bit errors |

# Document Control

| Version | Date | Author | Reviewer | Description |
| --- | --- | --- | --- | --- |
| 1.0 | 10-Sep-2026 | Student | — | Initial BCH architecture baseline |

## Document Status

☐ Draft    ☐ Under Review    ☐ Approved    ☐ Obsolete

## Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Student | Student | — | 10-Sep-2026 |

# 1. Purpose

Define the architecture of the BCH(15,7,2) error detection and correction engine and establish the approved structural baseline for RTL implementation, verification and FPGA implementation.

# 2. Architecture Overview

The architecture is a modular combinational BCH(15,7,2) engine. A 7-bit input is encoded into a 15-bit BCH codeword. An internal error-injection block can introduce zero, one or two bit errors for verification. The syndrome-based decoder calculates BCH syndromes using GF(16) arithmetic, determines the error pattern and corrects the received codeword. The corrected 7-bit data, corrected 15-bit codeword and error status are then provided at the top-level outputs.

# 3. ISA Definition

Not applicable. This project is a dedicated error-correction datapath and does not implement a processor instruction set architecture. The architectural data unit is BCH(15,7,2): 7 information bits, 8 parity bits, codeword length 15, and correction capability of up to two bit errors.

# 4. Top-Level Architecture

The top-level architecture contains four RTL blocks: BCH Encoder, Error Injector, BCH Decoder and Top-Level Integration. Data flows from data_in to the encoder, from the encoder to the error injector, and from the corrupted codeword to the decoder. The decoder produces corrected_codeword, corrected_data and error_status. All processing is combinational.

# 5. Datapath Architecture

The datapath begins with 7-bit data_in. The encoder appends eight zero bits and performs polynomial division using g(x)=x^8+x^7+x^6+x^4+1 to obtain the eight parity bits. The error injector XORs the codeword with one or two one-hot error masks when selected. The decoder evaluates S1 and S3 syndromes, performs GF(16) arithmetic, identifies error locations and flips the corresponding received bits. corrected_codeword[14:8] is presented as corrected_data.

# 6. Control Architecture

The error_type input controls the demonstration error condition: 00=no error, 01=single-bit error and 10=two-bit error. error_pos1 and error_pos2 select the bit positions used by the injector. The decoder independently determines the correction status from the received codeword. error_status is 00 for no error, 01 for one-bit correction, 10 for two-bit correction and 11 for an uncorrectable/unsupported condition.

# 7. Memory Architecture

No memory architecture is required. The BCH engine is a pure combinational datapath with no RAM, ROM, register file or external memory interface. All intermediate values are held in combinational logic and functions.

# 8. Architectural State

There is no sequential architectural state. The design has no clock, reset, program counter or stored state. The externally visible state at any instant is represented by the current input values and the combinational outputs: encoded_codeword, corrupted_codeword, corrected_codeword, corrected_data and error_status.

# 9. Interface Architecture

The top-level interface has data_in[6:0], error_type[1:0], error_pos1[3:0] and error_pos2[3:0] as inputs. Outputs are encoded_codeword[14:0], corrupted_codeword[14:0], corrected_codeword[14:0], corrected_data[6:0] and error_status[1:0]. FPGA pin assignment is provided by the Basys 3 XDC constraints.

# 10. Architectural Trade-offs

A syndrome-based decoder was selected instead of brute-force comparison against all valid codewords. The syndrome approach substantially reduces FPGA logic usage while retaining the required one- and two-bit correction capability. The architecture remains combinational for simple integration and direct observation of outputs. The trade-off is a longer combinational path through GF(16) arithmetic, which is reflected in timing analysis.

# 11. Supported Operation Matrix

| Instruction / Group | Format | Supported? | Architectural Behavior | Notes |
| --- | --- | --- | --- | --- |
| BCH Encoding | 7-bit → 15-bit | Yes | Generate BCH codeword with 8 parity bits | g(x)=x^8+x^7+x^6+x^4+1 |
| No Error | error_type=00 | Yes | Pass encoded word unchanged | Decoder status 00 |
| Single Error | error_type=01 | Yes | Inject and correct one bit | Decoder status 01 |
| Two Errors | error_type=10 | Yes | Inject and correct two bits | Decoder status 10 |
| Three+ Errors | More than 2 errors | No | Not guaranteed to correct | Outside BCH(15,7,2) capability |
| GF(16) Arithmetic | Internal decoder operations | Yes | Multiply, power and inverse operations | Used for syndrome decoding |
| Error Status | 2-bit status | Yes | Indicate correction condition | 00/01/10/11 |

The supported operating conditions are summarized below. Three-or-more-bit error correction is intentionally outside the supported architecture.

# 12. Architecture Block Register

| Block ID | Block Name | Purpose | Inputs | Outputs | Owner |
| --- | --- | --- | --- | --- | --- |
| BLK-01 | BCH Encoder | Generate 15-bit BCH codeword | data_in[6:0] | encoded_codeword[14:0] | bch_encoder.sv |
| BLK-02 | Error Injector | Inject 0/1/2 bit errors | encoded_codeword, error_type, error_pos1, error_pos2 | corrupted_codeword[14:0] | error_injector.sv |
| BLK-03 | BCH Decoder | Calculate syndromes and correct errors | received_codeword[14:0] | corrected_codeword, corrected_data, error_status | bch_decoder.sv |
| BLK-04 | Top Integration | Connect all blocks | All top-level inputs | All top-level outputs | bch_top.sv |
| BLK-05 | Simulation TB | Verify supported cases | Test vectors | Waveform/results | bch_tb.sv |
| BLK-06 | FPGA Constraints | Assign physical pins and timing | Top-level ports | Implementation constraints | bch_basys3.xdc / bch_timing.xdc |

The block register below identifies the major architectural components and their RTL ownership.

# 13. Architecture Decisions

| ADR-ID | Decision | Alternatives | Selected Solution | Rationale | Impact |
| --- | --- | --- | --- | --- | --- |
| ADR-01 | Code selection | Larger BCH codes; BCH(15,7,2) | BCH(15,7,2) | Matches project scope and supports correction of up to two errors | 15-bit codeword, 7 data bits |
| ADR-02 | Decoder method | Brute-force valid-codeword search; syndrome decoding | Syndrome decoding | Lower FPGA logic and more direct BCH algorithm | GF(16) combinational logic required |
| ADR-03 | Error demonstration | Manual corrupted values; internal injector | Internal error injector | Makes error injection part of the hardware datapath | Adds control inputs |
| ADR-04 | Datapath style | Sequential/pipelined; combinational | Combinational | Simple direct output and current project scope | Longer combinational path |
| ADR-05 | FPGA target | Other devices; Artix-7 | xc7a35tcpg236-1 | Matches Basys 3 target device | Requires Basys 3 XDC |
| ADR-06 | Timing analysis | 10 ns requirement; 15 ns requirement | 15 ns requirement | Current implemented design meets it with positive slack | WNS +1.286 ns |
| ADR-07 | Hardware validation | Physical board; software-only | Software/implementation validation | No physical Basys 3 board is available | Bitstream generated; physical validation pending |

The following decisions form the architectural baseline used for the current RTL and FPGA implementation.