RTL DESIGN SPECIFICATION

BCH(15,7,2) Hardware Error Detection and Correction Engine

Project: Design of a Hardware Error Detection and Correction Engine for Reliable Computing

| Document ID | BCH-RTL-004 |
| --- | --- |
| Document Title | RTL Design Specification |
| Project | BCH(15,7,2) Hardware Error Detection and Correction Engine |
| Document Version | 1.0 |
| Status | Complete / For Review |
| Author | Project Team |
| Reviewer | Project Guide |
| Tool Version | Vivado 2021.1 |
| Target Device | xc7a35tcpg236-1 / Basys 3 |
| HDL | SystemVerilog (.sv) |
| Top-Level Module | bch_top |

Target FPGA: AMD/Xilinx Artix-7 xc7a35tcpg236-1 (Basys 3)

Implementation tool: Vivado 2021.1 | HDL: SystemVerilog | Design style: Combinational RTL

# Document Control

| Version | Date | Author | Reviewer | Description |
| --- | --- | --- | --- | --- |
| 1.0 | 2026-09-10 | Project Team | Project Guide | Completed RTL specification for BCH(15,7,2) engine |
| 0.2 | 2026-08 | Project Team | Project Guide | Updated syndrome-based decoder and integration |
| 0.1 | 2026-08 | Project Team | Project Guide | Initial RTL structure and simulation setup |

## Document Status

☐ Draft    ☒ Under Review    ☐ Approved    ☐ Obsolete

## Approval: Project Guide / Department Review

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By | Project Team | Pending | 2026-09-10 |
| Reviewed By | Project Guide | Pending | — |
| Approved By | Department | Pending | — |
| Verification | Project Team | Complete for simulation | 2026-09-10 |

# 1. Purpose

This specification defines the synthesizable SystemVerilog RTL implementation of the BCH(15,7,2) hardware error detection and correction engine. The RTL maps the approved BCH architecture into encoder, error-injection, decoder and top-level integration modules. The design accepts 7-bit data, produces a 15-bit BCH codeword, optionally introduces zero/one/two bit errors for controlled verification, and uses syndrome-based decoding to correct up to two bit errors and recover the original 7-bit data.

# 2. RTL Hierarchy

The RTL hierarchy is: bch_top → bch_encoder, error_injector and bch_decoder. bch_top is the synthesis top level. bch_encoder generates the systematic BCH(15,7,2) codeword using the generator polynomial g(x)=x^8+x^7+x^6+x^4+1. error_injector creates controlled 0-, 1- or 2-bit error cases. bch_decoder calculates BCH syndromes, determines the error locator information, identifies error positions and produces the corrected codeword, recovered data and error status.

# 3. Module Responsibilities

bch_encoder.sv: encode 7-bit input data into a 15-bit BCH codeword. error_injector.sv: apply zero, one or two selectable bit flips to the encoded codeword for verification. bch_decoder.sv: calculate syndromes and perform syndrome-based correction for up to two errors. bch_top.sv: structurally connect all RTL blocks and expose the complete project interface. bch_tb.sv is a simulation-only verification testbench and is not synthesized.

# 4. Coding Standards

SystemVerilog synthesizable constructs are used. logic is used for RTL signals; always_comb is used for combinational processes; case statements are used for control selection; loops are used only where synthesis supports the required bounded operations. Signal names are descriptive and use lower_snake_case. No delays, file I/O, force/release, or testbench-only constructs are present in synthesizable modules. Decoder finite searches over the 15 valid BCH positions are bounded and synthesizable.

# 5. Clock / Reset Strategy

The current BCH engine is a fully combinational design. It contains no clock input, no reset input, and no sequential storage elements. Inputs propagate through encoder, error injector and decoder logic to the outputs. The timing analysis uses a virtual clock only to express a 15 ns timing requirement for static timing analysis; this virtual clock is not a physical clock in the RTL and does not make the design sequential.

# 6. Interface Definitions

bch_top exposes 7-bit data_in, 2-bit error_type, two 4-bit error-position controls, 15-bit encoded_codeword, 15-bit corrupted_codeword, 15-bit corrected_codeword, 7-bit corrected_data and 2-bit error_status. error_type=00 means no error, 01 means one-bit error, 10 means two-bit error. error_status=00 indicates no error, 01 indicates one-bit correction, 10 indicates two-bit correction, and 11 represents uncorrectable/unsupported conditions.

# 7. Parameterization

The implementation is intentionally fixed to BCH(15,7,2) for the project requirement. Data width is 7 bits, codeword width is 15 bits, parity width is 8 bits, error-position width is 4 bits, and error-status width is 2 bits. These widths are explicit in the RTL rather than exposed as generic parameters. No memory depth, reset vector or clock-frequency parameter is used.

# 8. RTL Quality Requirements

The RTL shall be synthesizable, free of unintended latches and multiple drivers, and shall have all top-level signals driven. Combinational processes assign their outputs for all control cases. The decoder uses deterministic GF(16) arithmetic and syndrome processing. Vivado synthesis and implementation completed successfully for the selected Artix-7 target. The generated bitstream completed successfully. Any tool warning not affecting functional or timing closure must be reviewed rather than silently ignored.

# 9. Review Checklist

RTL review checks include: module/file names match the hierarchy; top-level ports match the XDC; all outputs are driven; no unintended clock or reset is introduced; encoder generator polynomial is correct; error injection is restricted to 0/1/2-bit cases; decoder syndrome logic supports up to two errors; error status encoding is consistent; testbench cases match supported functionality; RTL is synthesizable in Vivado 2021.1; timing and utilization reports are retained as implementation evidence.

# 10. RTL Module Register

| Module ID | SystemVerilog File | Purpose | Inputs | Outputs | Status |
| --- | --- | --- | --- | --- | --- |
| BCH-RTL-01 | bch_encoder.sv | Generate systematic BCH(15,7,2) codeword using g(x) | data_in[6:0] | codeword[14:0] | Implemented / Synthesizable |
| BCH-RTL-02 | error_injector.sv | Inject 0, 1 or 2 selectable bit errors | codeword_in[14:0], error_type[1:0], error_pos1[3:0], error_pos2[3:0] | corrupted_codeword[14:0] | Implemented / Synthesizable |
| BCH-RTL-03 | bch_decoder.sv | Syndrome calculation, error-location and correction | received_codeword[14:0] | corrected_data[6:0], corrected_codeword[14:0], error_status[1:0] | Implemented / Synthesizable |
| BCH-RTL-04 | bch_top.sv | Top-level structural integration | All project inputs | All project outputs | Implemented / Synthesizable |
| BCH-SIM-01 | bch_tb.sv | Simulation stimulus and functional checks | DUT inputs / test vectors | Waveform / observations | Simulation only |

The module register below identifies every RTL source used in the implementation and its synthesis/verification role.

# 11. Top-Level Port Specification

| Port | Dir | Width | Description | Clock/Reset Relation | Constraints |
| --- | --- | --- | --- | --- | --- |
| data_in | Input | 7 | Original user data | Combinational | SW0–SW6; LVCMOS33 |
| error_type | Input | 2 | 00=no error, 01=1-bit, 10=2-bit | Combinational | SW7–SW8; LVCMOS33 |
| error_pos1 | Input | 4 | First error position 0–14 | Combinational | SW9–SW12; LVCMOS33 |
| error_pos2 | Input | 4 | Second error position 0–14 | Combinational | SW13–SW15 + BTNC; LVCMOS33 |
| encoded_codeword | Output | 15 | Generated BCH codeword | Combinational | Basys 3 LEDs |
| corrupted_codeword | Output | 15 | Codeword after selected errors | Combinational | 7-seg + JA |
| corrected_codeword | Output | 15 | Decoder-corrected BCH codeword | Combinational | JA/JB/JC |
| corrected_data | Output | 7 | Recovered original 7-bit data | Combinational | JC + VGA Hsync |
| error_status | Output | 2 | 00 none, 01 one-bit, 10 two-bit, 11 unsupported | Combinational | VGA Vsync + USB TX |

The top-level interface contains 71 physical I/O signals after bit expansion. The Basys 3 XDC maps data and error controls to switches/buttons and maps result/status signals to LEDs, seven-segment/JA/JB/JC/VGA/USB pins. The RTL itself remains board-independent; the XDC provides the physical board mapping.

# 12. Coding / Naming Conventions

| Item | Convention | Example |
| --- | --- | --- |
| Module names | lower_snake_case | bch_encoder |
| Signal names | lower_snake_case | encoded_codeword |
| Bit ranges | Explicit [MSB:LSB] | data_in[6:0] |
| Combinational blocks | always_comb | always_comb begin ... end |
| Instances | descriptive + _inst | encoder_inst |
| Status constants | explicit binary encoding | 2'b01 = one-bit corrected |
| Files | one primary module per RTL source | bch_decoder.sv |

Use lower_snake_case for module signals and ports, descriptive module names, explicit bit ranges, logic data types, always_comb for combinational logic, and _inst suffix for instantiated module names. Constants and status encodings are written as explicit binary values where useful. Comments identify the BCH generator polynomial and major GF(16) decoding operations.

# 13. RTL Review Findings

| Finding ID | File/Line | Severity | Finding | Action | Closure |
| --- | --- | --- | --- | --- | --- |
| RTL-REV-01 | bch_encoder.sv | Medium | Generator-polynomial implementation must match BCH(15,7,2) | Checked against g(x)=x^8+x^7+x^6+x^4+1 and simulation | Closed |
| RTL-REV-02 | error_injector.sv | Low | Error positions must remain within 0–14 | Testbench uses valid BCH bit positions; unsupported controls are outside project scope | Closed |
| RTL-REV-03 | bch_decoder.sv | Medium | Decoder complexity and combinational path require timing review | Syndrome-based decoder selected; implementation timing checked | Closed |
| RTL-REV-04 | bch_decoder.sv | Low | 3-bit correction is not supported by BCH(15,7,2) requirement | Document as unsupported/uncorrectable status 11; do not add 3-bit support | Closed |
| RTL-REV-05 | bch_tb.sv | Low | Physical FPGA validation not available | Simulation and implementation evidence retained; physical validation marked pending | Closed for current scope |

The reviewed RTL hierarchy is complete for the BCH(15,7,2) requirement. Functional simulation has verified no-error, one-bit-error and two-bit-error correction cases. No physical-board validation was performed because a Basys 3 board was not available; therefore board-level observations are not claimed as completed.