DESIGN VERIFICATION PLAN

BCH(15,7,2) Hardware Error Detection and Correction Engine

| Document Field | Value |
| --- | --- |
| Document ID | BCH-DVP-005 |
| Project | Design of a Hardware Error Detection and Correction Engine for Reliable Computing |
| Design | BCH(15,7,2) Error Detection and Correction Engine |
| HDL | SystemVerilog |
| Tool | AMD/Xilinx Vivado 2021.1 / XSim |
| Target Device | Artix-7 xc7a35tcpg236-1 (Basys 3 target) |
| Verification Status | Complete for defined RTL functional scope / For Review |
| Physical Board | Not available; FPGA hardware validation is deferred |
| Timing Requirement | 15 ns virtual-clock requirement; latest WNS +1.286 ns, 0 failing endpoints |
| Power Evidence | Vivado implemented-netlist estimate: 0.12 W total, low confidence |

# Document Control

| Version | Date | Author | Reviewer | Description |
| --- | --- | --- | --- | --- |
| 1.0 | 10-Sep-2026 | Project Team | Project Guide | Initial BCH(15,7,2) verification plan completed from controlled template. |

# Document Status

☐ Draft    ☒ Under Review    ☐ Approved    ☐ Obsolete

# Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By | Project Team | Pending | 10-Sep-2026 |
| Reviewed By | Project Guide | Pending |  |
| Approved By | Project Guide / Department | Pending |  |
| Status | For Review | Pending |  |

# 1. Purpose

Define the verification strategy, environments, test levels, coverage goals, checkers and exit criteria for the BCH(15,7,2) hardware error detection and correction engine.

# 2. Verification Objectives

Demonstrate functional correctness of BCH encoding, controlled 0/1/2-bit error injection, syndrome-based correction, recovered data, error-status reporting and top-level integration.

# 3. Verification Scope

Unit verification of bch_encoder.sv, error_injector.sv and bch_decoder.sv.

Integration verification of bch_top.sv and signal propagation between all RTL blocks.

System-level RTL verification of the complete BCH(15,7,2) data/error/correction path.

Regression testing using the complete directed SystemVerilog testbench after RTL changes.

Waveform inspection and requirement/functional scenario coverage; code and assertion coverage are not numerically measured in the current flow.

# 4. Verification Environment

Vivado 2021.1 XSim is used with bch_tb.sv. The DUT is bch_top. RTL sources are bch_encoder.sv, error_injector.sv and bch_decoder.sv. The design is combinational and therefore requires no clock or reset stimulus. Outputs are checked against expected encoded, corrupted and corrected values.

# 5. Verification Strategy

Directed deterministic tests are used because the BCH(15,7,2) scope has clearly defined supported error cases. Tests cover no error, one-bit error and two-bit error correction, including multiple error-position combinations. Waveforms are inspected for encoded_codeword, corrupted_codeword, corrected_codeword, corrected_data and error_status.

# 6. Coverage Strategy

Functional scenario coverage target is 100% of the defined BCH(15,7,2) scenarios: 0-bit, 1-bit and 2-bit errors. Requirement coverage target is 100% of in-scope requirements. Numerical code coverage and assertion coverage are not claimed because those measurements were not run.

# 7. Regression Strategy

Rerun the complete directed testbench after RTL changes. All previously passing supported cases must continue to pass before accepting a modification.

# 8. Exit Criteria

All defined supported functional tests pass; corrected_data equals data_in; corrected_codeword equals encoded_codeword; error_status correctly identifies no error, one-bit correction and two-bit correction; synthesis and implementation complete; timing has no failing endpoints under the 15 ns requirement.

# 9. Requirement-to-Test Matrix

| REQ-ID | Test ID | Stimulus | Expected Result | Checker | Status |
| --- | --- | --- | --- | --- | --- |
| REQ-01 | TB-05 | 7-bit data input to BCH encoder | Valid 15-bit BCH(15,7,2) codeword generated | Waveform / reference check | PASS |
| REQ-02 | TB-01 | No injected error | Output remains unchanged; status 00 | Waveform comparison | PASS |
| REQ-03 | TB-02 | One-bit corruption | Original codeword and data recovered; status 01 | Waveform comparison | PASS |
| REQ-04 | TB-03 | Two-bit corruption | Original codeword and data recovered; status 10 | Waveform comparison | PASS |
| REQ-05 | TB-04 | Second two-bit position combination | Correction succeeds for positions 1 and 12; status 10 | Waveform comparison | PASS |
| REQ-06 | TB-06 | Error-type selection 00/01/10 | Only selected error mask is applied | Waveform comparison | PASS |
| REQ-07 | TB-07 | Error status output | Status distinguishes 0/1/2 supported errors | Waveform comparison | PASS |
| REQ-08 | TB-08 | Top-level integration | All encoder/injector/decoder outputs propagate correctly | RTL integration simulation | PASS |
| REQ-09 | TB-08 | Synthesizable RTL | Synthesis and implementation complete | Vivado reports | PASS |
| REQ-10 | TB-08 | Timing requirement | No failing endpoints; WNS +1.286 ns at 15 ns requirement | Static timing report | PASS |
| REQ-11 | TB-08 | 3-bit correction limitation | 3-bit correction is outside BCH(15,7,2) scope and is not claimed | Scope review | N/A |

# 10. Test Suite

| Test ID | Feature | Type | Priority | Expected Result | Status |
| --- | --- | --- | --- | --- | --- |
| TB-01 | No error | Directed functional | High | status 00; corrected_codeword = encoded_codeword; corrected_data = data_in | PASS |
| TB-02 | Single-bit error | Directed functional | High | 1-bit corruption is corrected; status 01 | PASS |
| TB-03 | Two-bit error | Directed functional | High | 2-bit corruption is corrected; status 10 | PASS |
| TB-04 | Second two-bit position combination | Directed functional | High | Errors at positions 1 and 12 are corrected; status 10 | PASS |
| TB-05 | Encoder module | Unit functional | High | Generated codeword follows BCH generator-polynomial encoding | PASS |
| TB-06 | Error injector module | Unit functional | Medium | No/one/two error modes alter only selected positions | PASS |
| TB-07 | Decoder status behavior | Unit functional | High | Status distinguishes 0, 1 and 2 supported errors | PASS |
| TB-08 | Top-level integration | Integration | High | All submodules are connected and outputs propagate correctly | PASS |
| TB-09 | Unsupported 3-bit capability | Negative/limitation | Medium | 3-bit correction is outside stated BCH(15,7,2) scope and is not claimed | NOT APPLICABLE |
| TB-10 | Implementation evidence | Implementation | High | Synthesis, implementation, timing and bitstream generation complete | PASS |
| TB-11 | Physical FPGA validation | Hardware validation | Medium | Deferred because no Basys 3 board is currently available | DEFERRED |

# 11. Coverage Goals

| Coverage Type | Target | Current | Gap | Closure Plan |
| --- | --- | --- | --- | --- |
| Requirement coverage | 100% of in-scope requirements | Mapped to REQ-01..REQ-11 | None for defined RTL scope | Maintain matrix after RTL changes |
| Functional scenarios | 100%: 0/1/2-bit cases | TB-01..TB-04 covered | None observed | Add directed vectors if decoder changes |
| Code coverage | Measure if required | Not measured | Percentage unavailable | Run coverage flow if required by project review |
| Assertion coverage | Measure if assertions are added | No numerical assertion coverage | Assertions not used | Add assertions only if verification scope expands |
| Regression | All defined supported tests pass | TB-01..TB-04 pass | None observed | Repeat full regression after RTL modifications |

# 12. Verification Environment Components

| Component | Purpose | Implementation | Status |
| --- | --- | --- | --- |
| bch_tb.sv | Generate deterministic stimuli and observe DUT | SystemVerilog testbench in XSim | Complete |
| bch_top.sv | Integrate encoder, injector and decoder | Structural RTL | Complete |
| bch_encoder.sv | Encode 7-bit data into 15-bit codeword | Combinational polynomial division | Verified |
| error_injector.sv | Inject 0/1/2-bit errors | XOR masks using error positions | Verified |
| bch_decoder.sv | Calculate syndromes and correct up to 2 bits | GF(16) syndrome/error-locator logic | Verified for defined tests |
| XSim | RTL simulation engine | Vivado 2021.1 simulator | Used |
| Waveform viewer | Inspect encoded/corrupted/corrected outputs | Vivado simulation waveform | Used |
| Vivado implementation | Synthesis, place, route, timing and power estimation | Vivado 2021.1 implementation flow | Complete |

# 13. Verification Risks

| Risk | Impact | Mitigation | Owner | Status |
| --- | --- | --- | --- | --- |
| Generator-polynomial implementation error | High | Verify encoded codeword against known BCH behavior and simulation vectors | Design Team | Closed for defined tests |
| Syndrome/decoder logic error | High | Directed 1-bit and multiple 2-bit error-position tests with waveform comparison | Design Team | Closed for defined tests |
| Invalid error positions | Medium | Use 4-bit positions within 0..14 in supported tests | Testbench | Controlled |
| Overclaiming 3-bit correction | High | Document BCH(15,7,2) limit; do not claim 3-bit correction | Project Team | Controlled |
| Timing constraint interpretation | Medium | Report WNS and requirement honestly; do not equate constraint change with physical speed increase | Project Team | Controlled |
| Low-confidence power estimate | Medium | Identify Vivado estimate as implementation-based low-confidence result; avoid claiming measured board power | Project Team | Open / documented |
| No physical Basys 3 board | Medium | Mark hardware validation as deferred; report only RTL/implementation evidence | Project Team | Deferred |
| RTL modification regression failure | Medium | Rerun complete directed regression after every RTL change | Design Team | Ongoing |