DESIGN VERIFICATION REPORT

BCH(15,7,2) Hardware Error Detection and Correction Engine

| Document ID | BCH-DVR-006 |
| --- | --- |
| Document Title | Design Verification Report |
| Project | BCH(15,7,2) Hardware Error Detection and Correction Engine |
| Document Version | 1.0 |
| Status | Complete / For Review |
| Author | Project Team |
| Reviewer | Project Guide |
| Simulator | Vivado 2021.1 / XSim |
| Target Device | xc7a35tcpg236-1 / Basys 3 |
| Testbench | bch_tb.sv |
| Top-Level Module | bch_top |

# Document Control

| Version | Date | Author | Reviewer | Description |
| --- | --- | --- | --- | --- |
| 1.0 | 2026-09-10 | Project Team | Project Guide | Completed BCH verification report with actual simulation and implementation results |
| 0.2 | 2026-08 | Project Team | Project Guide | Updated syndrome-based decoder verification |
| 0.1 | 2026-08 | Project Team | Project Guide | Initial verification baseline |

## Document Status

☐ Draft    ☒ Under Review    ☐ Approved    ☐ Obsolete

## Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By | Project Team | Pending | 2026-09-10 |
| Reviewed By | Project Guide | Pending | — |
| Approved By | Department | Pending | — |
| Verification | Project Team | Complete for simulation | 2026-09-10 |

# 1. Purpose

Record executed verification activities, results, failures, coverage and closure evidence for the BCH(15,7,2) hardware error detection and correction engine.

# 2. Verification Baseline

RTL revision: BCH RTL v1.0 | Simulator: Vivado 2021.1 XSim | Testbench: bch_tb.sv | DUT: bch_top | Target device: xc7a35tcpg236-1 | Date: 2026-09-10.

# 3. Summary

Four directed functional tests were executed: TB-01 no error, TB-02 one-bit error, TB-03 two-bit error and TB-04 an additional two-bit position combination. All four tests passed. No functional test failed or was blocked. Code coverage percentages were not measured in the current flow.

# 4. Test Results

For data_in=5A, the encoder generated 5ABC. TB-02 corrupted the word to 5AB4 and the decoder restored 5ABC, recovered data 5A and produced status 01. TB-03 corrupted the word to 4ABE and restored 5ABC, recovered data 5A and produced status 10. TB-04, using error positions 1 and 12, also corrected successfully.

# 5. Failure Analysis

No functional failure was observed in the supported test cases, so no functional defect was opened. Three-bit correction is outside the BCH(15,7,2) project scope and is therefore not treated as a failed test.

# 6. Coverage Results

Functional scenario coverage includes no-error, one-bit-error and two-bit-error operation, with more than one valid two-bit position combination exercised. Statement, branch and toggle coverage were not measured in the current Vivado flow; no numerical code-coverage value is claimed.

# 7. Requirement Closure

The supported requirements are closed using objective simulation and implementation evidence. BCH encoding, no-error operation, one-bit correction, two-bit correction, top-level integration, synthesis and the selected 15 ns timing requirement were verified.

# 8. Verification Conclusion

The defined RTL verification exit criteria are met. All supported directed tests passed, the original data and codeword were recovered, synthesis and implementation completed, and the latest implementation reported WNS +1.286 ns, TNS 0.000 ns and 0 failing endpoints. Physical Basys 3 validation remains pending because no board was available.

# 9. Test Execution Summary

| Test ID | Date | RTL Rev | Result | Waveform/Log | Defect ID |
| --- | --- | --- | --- | --- | --- |
| TB-01 | 2026-09-10 | v1.0 | PASS | No error: data 5A; encoded 5ABC; corrected 5ABC; data 5A; status 00 | — |
| TB-02 | 2026-09-10 | v1.0 | PASS | 1-bit: corrupted 5AB4; corrected 5ABC; data 5A; status 01 | — |
| TB-03 | 2026-09-10 | v1.0 | PASS | 2-bit: corrupted 4ABE; corrected 5ABC; data 5A; status 10 | — |
| TB-04 | 2026-09-10 | v1.0 | PASS | 2-bit positions 1 and 12: corrected successfully; status 10 | — |

# 10. Defect Log

| Defect ID | Severity | Description | Root Cause | Fix | Retest | Status |
| --- | --- | --- | --- | --- | --- | --- |
| D-001 | N/A | No functional failure in supported tests | None | No corrective action required | N/A | Closed |
| D-002 | N/A | Three-bit correction not supported | Project scope is BCH(15,7,2) | Document as unsupported; no RTL change | N/A | Closed |
| D-003 | Low | Physical board validation unavailable | Basys 3 board not available | Report simulation/implementation evidence only | Pending hardware | Open |

# 11. Coverage Summary

| Coverage Type | Target | Achieved | Waiver/Exclusion | Status |
| --- | --- | --- | --- | --- |
| Functional scenarios | No error, 1-bit and 2-bit cases | All defined scenarios executed | None | PASS |
| No-error | No-error operation | Executed in TB-01 | None | PASS |
| One-bit correction | One-bit correction | Executed in TB-02 | None | PASS |
| Two-bit correction | Two-bit correction | Executed in TB-03 and TB-04 | None | PASS |
| Code coverage | Statement/branch/toggle | Not measured | Not measured in current flow | N/A |
| Physical validation | Basys 3 hardware test | Not performed | Board unavailable | PENDING |

# 12. Requirement Closure Matrix

| REQ-ID | Verification Evidence | Result | Open Issue |
| --- | --- | --- | --- |
| REQ-01 | TB-01 verified BCH(15,7,2) encoding; data 5A produced codeword 5ABC | PASS | None |
| REQ-02 | TB-01 verified no-error operation; corrected codeword remained 5ABC; status 00 | PASS | None |
| REQ-03 | TB-02 verified 5AB4 corrected to 5ABC; corrected_data=5A; status 01 | PASS | None |
| REQ-04 | TB-03/TB-04 verified two-bit correction and corrected_data=5A; status 10 | PASS | None |
| REQ-05 | bch_top integration verified through simulation and successful implementation | PASS | None |
| REQ-06 | Latest STA: WNS +1.286 ns, TNS 0.000 ns, 0 failing endpoints under 15 ns requirement | PASS | None |
| REQ-07 | Three-bit correction is outside BCH(15,7,2) project scope | N/A | Not a project requirement |