SYNTHESIS REPORT

BCH(15,7,2) Hardware Error Detection and Correction Engine

| Document ID | BCH-SYN-007 |
| --- | --- |
| Document Title | Synthesis Report |
| Project | BCH(15,7,2) Hardware Error Detection and Correction Engine |
| Document Version | 1.0 |
| Status | Complete / For Review |
| Author | Project Team |
| Reviewer | Project Guide |
| Tool | Vivado 2021.1 |
| Target Device | xc7a35tcpg236-1 / Basys 3 |
| Top-Level Module | bch_top |
| HDL | SystemVerilog |

# Document Control

| Version | Date | Author | Reviewer | Description |
| --- | --- | --- | --- | --- |
| 1.0 | 2026-09-10 | Project Team | Project Guide | Completed BCH synthesis report with actual implementation results |
| 0.2 | 2026-08 | Project Team | Project Guide | Updated syndrome-based decoder implementation |
| 0.1 | 2026-08 | Project Team | Project Guide | Initial synthesis baseline |

## Document Status

☐ Draft    ☒ Under Review    ☐ Approved    ☐ Obsolete

## Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By | Project Team | Pending | 2026-09-10 |
| Reviewed By | Project Guide | Pending | — |
| Approved By | Department | Pending | — |
| Synthesis | Project Team | Complete | 2026-09-10 |

# 1. Purpose

Record the synthesis results for the BCH(15,7,2) hardware error detection and correction engine, including RTL synthesis status, inferred hardware resources, warnings, and synthesis closure.

# 2. Synthesis Baseline

RTL revision: BCH RTL v1.0 | Synthesis tool: Vivado 2021.1 | HDL: SystemVerilog | Top module: bch_top | Target: xc7a35tcpg236-1.

# 3. Synthesis Configuration

The BCH design was synthesized for the AMD/Xilinx Artix-7 xc7a35tcpg236-1 device. The top-level module is bch_top. The design is combinational and has no physical clock or reset in the RTL. The project XDC contains the Basys 3 I/O assignments and the timing requirement used for implementation analysis.

# 4. Synthesis Results

Synthesis completed successfully for the BCH RTL hierarchy. The selected syndrome-based decoder implementation is used. The latest implementation resource result reports 65 LUTs out of 20,800 available LUTs, corresponding to approximately 0.31% LUT utilization.

# 5. Resource Utilization

The implemented design uses 65 LUTs (0.31% of available LUTs). The top-level interface expands to 71 physical I/O signals, using 71 of 106 available I/O resources, approximately 66.98%. The large I/O percentage is caused by the deliberately exposed BCH inputs and multiple 15-bit result/status outputs for the Basys 3 demonstration mapping.

# 6. Synthesis Quality and Warnings

The synthesis/implementation flow completed successfully. Any Vivado informational messages or warnings that do not prevent synthesis or implementation are not treated as functional failures. No fabricated warning count is reported because a consolidated warning count was not retained as a project result.

# 7. Timing-Relevant Synthesis Observations

The latest implementation achieved WNS +1.286 ns and TNS 0.000 ns under the selected 15 ns timing requirement, with 0 failing endpoints. The corresponding estimated worst data-path delay is approximately 13.714 ns (15.000 ns minus 1.286 ns). This is a timing-analysis result, not a change to the RTL clocking model.

# 8. Power-Relevant Synthesis/Implementation Result

| Resource | Used |
| --- | --- |
| LUT | 65 (0.31%) |
| I/O | 71 / 106 (66.98%) |
| Slice | 22 / 8150 (0.27%) |
| LUT as Logic | 65 / 20800 (0.31%) |
| Flip-Flop | 0 (combinational design) |
| BRAM | 0 |
| DSP | 0 |
| Clocking | No physical clock used |
| Slice LUT utilization | 65 LUTs |
| Top-level cells | 158 |

Vivado reported a total power estimate of 0.12 W, consisting of approximately 0.048 W dynamic and 0.072 W static power. The report is a tool estimate with low confidence and is not a physical-board power measurement.

# 9. Bitstream Generation

| Item | Result | Status | Utilization % | Target | Status |
| --- | --- | --- | --- | --- | --- |
| Synthesis | Completed successfully | PASS | 0.31% LUT | Low resource use | PASS |
| Implementation | Completed successfully | PASS | 71 I/O used | 71/106 I/O | PASS |
| Timing | WNS +1.286 ns; TNS 0; 0 failing endpoints | PASS | — | 15 ns requirement | PASS |
| Hold timing | WHS +1.906 ns; THS 0; 0 failing endpoints | PASS | — | No hold violation | PASS |
| Bitstream | Write Bitstream Complete | PASS | — | Bitstream generated | PASS |
| Power estimate | 0.12 W total; 0.048 W dynamic; 0.072 W static | INFO | — | Tool estimate | INFO |
| DRC | Completed with 0 errors | PASS | — | No blocking DRC issue | PASS |
| Physical FPGA test | Not performed; Basys 3 board unavailable | PENDING | — | Hardware validation pending | PENDING |

The Vivado write-bitstream step completed successfully. Therefore a programming bitstream was generated for the selected Artix-7/Basys 3 target. Physical programming and hardware observation were not performed because the physical Basys 3 board was unavailable.

# 10. Synthesis Conclusion

| Item | Observation | Disposition | BRAM | DSP | Notes |
| --- | --- | --- | --- | --- | --- |
| Synthesis | No blocking synthesis issue reported | Closed | 0 | 0 | RTL synthesized successfully |
| Implementation | Implementation completed | Closed | 0 | 0 | No blocking implementation issue |
| Timing | All user-specified timing constraints met | Closed | — | — | WNS +1.286 ns |
| Hold | All hold constraints met | Closed | — | — | WHS +1.906 ns |
| DRC | 0 errors reported | Closed | — | — | Design-rule check passed |
| Power | Low-confidence Vivado estimate | Recorded | — | — | Total 0.12 W |
| Physical validation | No Basys 3 board available | Pending | — | — | Not claimed as completed |
| Bitstream | Generated successfully | Closed | — | — | Write Bitstream Complete |
| Target device | xc7a35tcpg236-1 | Recorded | — | — | Artix-7 / Basys 3 target |
| Top module | bch_top | Recorded | — | — | BCH top-level RTL |
| HDL | SystemVerilog | Recorded | — | — | Vivado 2021.1 |
| Decoder | Syndrome-based BCH decoder | Recorded | — | — | Chosen implementation |

The BCH RTL synthesized and implemented successfully. Resource utilization is low at 65 LUTs (0.31%), timing closure is achieved under the 15 ns requirement with WNS +1.286 ns, and bitstream generation completed successfully. Physical FPGA validation remains pending.

# 11. Critical Warnings / Errors

| ID | Severity | Message | Root Cause | Action | Status |
| --- | --- | --- | --- | --- | --- |
| SYN-01 | Info | Synthesis completed successfully | Normal Vivado synthesis flow | No action required | Closed |
| IMP-01 | Info | Implementation completed successfully | Normal implementation flow | No action required | Closed |
| TIM-01 | Info | WNS +1.286 ns under 15 ns requirement | Combinational timing constraint | No action required | PASS |
| TIM-02 | Info | WHS +1.906 ns | Hold analysis | No action required | PASS |
| DRC-01 | Info | DRC finished with 0 errors | Design-rule check | No action required | PASS |
| PWR-01 | Warning | Power confidence reported as Low | Vectorless/tool-estimated activity | Record estimate; do not claim physical measurement | Recorded |
| SIM-01 | Error | SAIF logging attempt found no matching objects | SAIF logging command did not match the simulation hierarchy | Not used for final power claim | Closed |
| FPGA-01 | Pending | Physical FPGA validation not performed | No Basys 3 board available | Perform hardware test when board is available | Pending |
| BCH-01 | Limitation | Design supports 0, 1 and 2-bit error cases | BCH(15,7,2) correction capability | Do not claim 3-bit correction | Documented |
| PWR-02 | Info | Total estimated on-chip power = 0.12 W | Vivado power analysis | Use as implementation estimate | Recorded |

# 12. Synthesis Comparison

| Revision | LUT | FF | BRAM | Fmax | Notes |
| --- | --- | --- | --- | --- | --- |
| Current | 65 | 0 | 0 | N/A | Syndrome-based decoder; latest implemented result |
| RTL architecture | — | — | — | N/A | Combinational BCH(15,7,2) architecture |
| Target | — | — | — | N/A | Artix-7 xc7a35tcpg236-1 |
| Timing result | — | — | — | 72.9 MHz equivalent | Derived from ~13.714 ns worst data-path delay; not a measured clock |
| Power estimate | — | — | — | — | 0.12 W total; low confidence |
| I/O result | — | — | — | — | 71 of 106 I/O used |
| Bitstream | — | — | — | — | Generated successfully |
| Validation | — | — | — | — | Physical Basys 3 validation pending |