STATIC TIMING ANALYSIS REPORT

BCH(15,7,2) Error Detection and Correction Engine

| Document Field | Value |
| --- | --- |
| Document ID | BCH-STA-08 |
| Project | BCH(15,7,2) Error Detection and Correction Engine |
| Document Type | Static Timing Analysis Report |
| Tool | Xilinx Vivado 2021.1 |
| Target Device | xc7a35tcpg236-1 |
| Top Module | bch_top |
| Design Type | Combinational RTL |
| Timing Constraint | virtual_clk, 15.000 ns |
| Worst Setup Slack | +1.286 ns |
| Worst Hold Slack | +1.906 ns |

# Document Control

| Version | Date | Author | Reviewer | Description |
| --- | --- | --- | --- | --- |
| 1.0 | 10-Sep-2026 | Project Team | Guide / Reviewer | Initial BCH static timing analysis report |
| 1.1 | 10-Sep-2026 | Project Team | — | Updated with final post-implementation timing results |

## Document Status

☐ Draft    ☐ Under Review    ☐ Approved    ☐ Obsolete

## Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By | Project Team |  | 10-Sep-2026 |
| Reviewed By | — |  | — |
| Approved By | — |  | — |

# 1. Purpose

Record the timing constraints and post-implementation static timing results for the BCH(15,7,2) error detection and correction engine implemented in Vivado 2021.1.

# 2. Clock Definition

The design is combinational and has no functional clock or sequential state. For static timing analysis, the project uses the virtual clock virtual_clk with a 15.000 ns period.

# 3. Constraint Validation

The virtual clock and input/output timing constraints were recognized by Vivado. The timing report shows 9 setup and 9 hold endpoints with no unconstrained timing endpoints.

# 4. Setup Analysis

The implemented design has WNS = +1.286 ns, TNS = 0.000 ns, and 0 failing setup endpoints. The worst setup path is error_pos2[2] to error_status[1], with 13.689 ns total delay against the 15.000 ns requirement.

# 5. Hold Analysis

The implemented design has WHS = +1.906 ns, THS = 0.000 ns, and 0 failing hold endpoints. No hold timing violation is reported.

# 6. Critical Path Analysis

The critical setup path is from error_pos2[2] to error_status[1]. Its total delay is 13.689 ns, consisting of 5.473 ns logic delay and 8.216 ns net delay. The path contains 6 logic levels and has high fanout of 31.

# 7. Timing Closure Actions

The final implementation was analyzed with the 15.000 ns timing requirement. The resulting positive WNS and zero failing endpoints show timing closure was achieved. No additional pipelining was required because the BCH RTL is a combinational design.

# 8. Timing Conclusion

All user-specified timing constraints are met. WNS is +1.286 ns, TNS is 0.000 ns, WHS is +1.906 ns, THS is 0.000 ns, and there are no failing endpoints.

# 9. Clock Summary

| Clock | Period ns | Frequency MHz | Source | Status |
| --- | --- | --- | --- | --- |
| virtual_clk | 15.000 | 66.667 | Virtual clock defined in bch_timing.xdc | PASS |

# 10. Timing Summary

| Metric | Value | Requirement | Status |
| --- | --- | --- | --- |
| Clock Period | 15.000 ns | 15.000 ns | PASS |
| Worst Setup Slack (WNS) | +1.286 ns | >= 0 ns | PASS |
| Total Setup Slack (TNS) | 0.000 ns | 0 ns minimum | PASS |
| Setup Failing Endpoints | 0 | 0 | PASS |
| Worst Hold Slack (WHS) | +1.906 ns | >= 0 ns | PASS |
| Total Hold Slack (THS) | 0.000 ns | 0 ns minimum | PASS |
| Hold Failing Endpoints | 0 | 0 | PASS |
| Worst Setup Path Delay | 13.689 ns | <= 15.000 ns | PASS |
| Timing Endpoints | 9 setup / 9 hold | All constrained | PASS |
| Overall Timing Status | All constraints met | Timing closure required | PASS |

# 11. Critical Paths

| Path ID | Startpoint | Endpoint | Delay ns | Slack ns | Logic / Cause | Action |
| --- | --- | --- | --- | --- | --- | --- |
| Path 1 | error_pos2[2] | error_status[1] | 13.689 | +1.286 | 5.473 logic + 8.216 net; 6 levels; fanout 31 | Final path; timing met |
| Path 2 | error_pos2[2] | error_status[0] | 11.809 | +3.166 | 5.433 logic + 6.376 net; 4 levels; fanout 28 | Timing met |
| Path 3 | data_in[6] | corrected_data[6] | 7.862 | +7.113 | 4.946 logic + 2.916 net; 2 levels | Timing met |
| Path 4 | data_in[2] | corrected_data[2] | 7.416 | +7.559 | 4.995 logic + 2.421 net; 2 levels | Timing met |
| Path 5 | data_in[3] | corrected_data[3] | 7.384 | +7.591 | 4.967 logic + 2.416 net; 2 levels | Timing met |
| Path 6 | data_in[4] | corrected_data[4] | 7.338 | +7.637 | 4.947 logic + 2.391 net; 2 levels | Timing met |
| Path 7 | data_in[5] | corrected_data[5] | 7.306 | +7.669 | 4.966 logic + 2.339 net; 2 levels | Timing met |
| Path 8 | data_in[0] | corrected_data[0] | 7.298 | +7.677 | 4.945 logic + 2.352 net; 2 levels | Timing met |
| Path 9 | data_in[1] | corrected_data[1] | 7.170 | +7.805 | 4.975 logic + 2.195 net; 2 levels | Timing met |

# 12. Timing Closure Log

| Revision | Problem | Action | WNS | TNS | Result |
| --- | --- | --- | --- | --- | --- |
| Final | Worst setup path had limited positive margin | Analyzed final implementation under 15.000 ns virtual-clock requirement | +1.286 | 0.000 | PASS — timing constraints met |
| Final | Hold timing verification | Checked post-implementation hold timing | +1.906 | 0.000 | PASS — no hold violations |
| Final | Timing endpoint validation | Verified constrained setup/hold endpoints | +1.286 | 0.000 | PASS — 0 failing endpoints |

Final Result: The BCH(15,7,2) implementation meets the specified 15.000 ns static timing requirement. Vivado 2021.1 reports WNS +1.286 ns, TNS 0.000 ns, WHS +1.906 ns, THS 0.000 ns, and zero failing endpoints. The 13.689 ns worst reported path delay is below the 15.000 ns requirement. Because the RTL is combinational, the reported virtual-clock frequency is a timing-analysis reference, not a measured FPGA operating frequency.