FPGA VALIDATION REPORT

BCH(15,7,2) Hardware Error Detection and Correction Engine

| Document Field | Value |
| --- | --- |
| Document ID | BCH-FVR-009 |
| Document Title | FPGA Validation Report |
| Project | BCH(15,7,2) Hardware Error Detection and Correction Engine |
| Version | 1.0 |
| Status | Complete for pre-hardware validation / For Review |
| Tool | Vivado 2021.1 |
| Target Board | Digilent Basys 3 |
| Target Device | xc7a35tcpg236-1 |
| Top Module | bch_top |
| Physical Board | Not available |

# Document Control

| Version | Date | Author | Reviewer | Description |
| --- | --- | --- | --- | --- |
| 1.0 | 10-Sep-2026 | Project Team | Project Guide | Completed BCH FPGA validation report; physical validation marked pending. |
| 0.2 | 2026-08 | Project Team | Project Guide | Updated implementation and bitstream evidence. |
| 0.1 | 2026-08 | Project Team | Project Guide | Initial FPGA validation baseline. |

## Document Status

☐ Draft    ☒ Under Review    ☐ Approved    ☐ Obsolete

## Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By | Project Team | Pending | 10-Sep-2026 |
| Reviewed By | Project Guide | Pending | — |
| Approved By | Department | Pending | — |
| Hardware Validation | Project Team | Pending board availability | — |

# 1. Purpose

Document the planned Basys 3 hardware bring-up, FPGA configuration, I/O mapping and validation status for the BCH(15,7,2) error detection and correction engine. RTL simulation and Vivado implementation evidence are available; physical board validation is pending because the Basys 3 board is not available.

# 2. Target Board

Target board: Digilent Basys 3. FPGA device: Artix-7 xc7a35tcpg236-1. The design uses the board's 3.3 V LVCMOS33 I/O standard. No physical-board clock is used by the combinational BCH RTL; the 15 ns virtual clock is used only for static timing analysis.

# 3. Bitstream Information

Vivado project: bch_error_correction. Top module: bch_top. RTL revision: BCH RTL v1.0. Bitstream generation completed successfully in Vivado 2021.1. A checksum was not recorded in the available project evidence, so no checksum is invented. Date: 10-Sep-2026.

# 4. Hardware Bring-Up

Physical bring-up was not performed because no Basys 3 board was available. Therefore power-on, programming, physical clock, reset, LED, seven-segment, VGA, USB and connector observations are pending. The generated bitstream is available for later board programming.

# 5. Validation Strategy

Simulation results are mapped to the observable top-level I/O. The functional cases are no error, one-bit error and two-bit error correction. Physical acceptance is deliberately separated from RTL/implementation verification so that no unperformed board result is claimed.

# 6. Results

No physical measurements were obtained. RTL simulation passed the defined supported cases, synthesis and implementation completed, timing closed with WNS +1.286 ns and 0 failing endpoints, and bitstream generation completed. Hardware observations remain pending.

# 7. Deviations / Issues

There is no simulation-versus-hardware deviation to report because hardware execution was not performed. The only open validation item is physical Basys 3 testing. The power result of 0.12 W is a low-confidence Vivado implementation estimate, not a board measurement.

# 8. Conclusion

RTL and implementation acceptance criteria for the current scope are met. FPGA hardware acceptance cannot yet be declared because the physical Basys 3 validation has not been performed.

# 9. Board Configuration

| Item | Value |
| --- | --- |
| Board | Digilent Basys 3 |
| FPGA | Artix-7 xc7a35tcpg236-1 |
| I/O Voltage | 3.3 V |
| I/O Standard | LVCMOS33 |
| Top-Level Module | bch_top |
| RTL | SystemVerilog |
| Vivado Version | 2021.1 |
| Bitstream | Generated successfully |
| Physical Programming | Not performed; board unavailable |
| Physical Clock | Not applicable to combinational RTL |

The intended board configuration is Basys 3 with Artix-7 xc7a35tcpg236-1, 3.3 V LVCMOS33 I/O and the supplied BCH Basys 3 XDC. Inputs are assigned to switches and button as specified; encoded/corrupted/corrected outputs are routed to LEDs, seven-segment, JA/JB/JC, VGA and USB-related pins.

# 10. Pin / I/O Assignment

| Signal | FPGA Pin | I/O Standard | Direction | Function | Status |
| --- | --- | --- | --- | --- | --- |
| data_in[0] | SW0 | LVCMOS33 | Input | Data bit 0 | Not physically tested |
| data_in[1] | SW1 | LVCMOS33 | Input | Data bit 1 | Not physically tested |
| data_in[2] | SW2 | LVCMOS33 | Input | Data bit 2 | Not physically tested |
| data_in[3] | SW3 | LVCMOS33 | Input | Data bit 3 | Not physically tested |
| data_in[4] | SW4 | LVCMOS33 | Input | Data bit 4 | Not physically tested |
| data_in[5] | SW5 | LVCMOS33 | Input | Data bit 5 | Not physically tested |
| data_in[6] | SW6 | LVCMOS33 | Input | Data bit 6 | Not physically tested |
| error_type[0] | SW7 | LVCMOS33 | Input | Error type bit 0 | Not physically tested |
| error_type[1] | SW8 | LVCMOS33 | Input | Error type bit 1 | Not physically tested |
| error_pos1[0] | SW9 | LVCMOS33 | Input | Error position 1 bit 0 | Not physically tested |
| error_pos1[1] | SW10 | LVCMOS33 | Input | Error position 1 bit 1 | Not physically tested |
| error_pos1[2] | SW11 | LVCMOS33 | Input | Error position 1 bit 2 | Not physically tested |
| error_pos1[3] | SW12 | LVCMOS33 | Input | Error position 1 bit 3 | Not physically tested |
| error_pos2[0] | SW13 | LVCMOS33 | Input | Error position 2 bit 0 | Not physically tested |
| error_pos2[1] | SW14 | LVCMOS33 | Input | Error position 2 bit 1 | Not physically tested |
| error_pos2[2] | SW15 | LVCMOS33 | Input | Error position 2 bit 2 | Not physically tested |
| error_pos2[3] | BTNC | LVCMOS33 | Input | Error position 2 bit 3 | Not physically tested |
| encoded_codeword[14:0] | LED0–LED14 | LVCMOS33 | Output | Encoded BCH codeword | Not physically tested |
| corrupted_codeword[14:0] | 7-seg + JA | LVCMOS33 | Output | Corrupted codeword display/interface | Not physically tested |
| corrected_codeword[14:0] | JA/JB/JC | LVCMOS33 | Output | Corrected codeword interface | Not physically tested |
| corrected_data[6:0] | JC + VGA Hsync | LVCMOS33 | Output | Recovered 7-bit data | Not physically tested |
| error_status[1:0] | VGA Vsync + USB TX | LVCMOS33 | Output | 0/1/2-bit correction status | Not physically tested |

The supplied Basys 3 XDC defines all 71 top-level I/O connections. The mapping is documented below; no pin is claimed as physically tested.

# 11. Hardware Test Matrix

| Test ID | Feature | Expected | Observed | Evidence | Status |
| --- | --- | --- | --- | --- | --- |
| HT-01 | Power-on | Board powers without issue | Not performed | Basys 3 unavailable | PENDING |
| HT-02 | FPGA programming | Generated bitstream programs successfully | Not performed | No physical board | PENDING |
| HT-03 | Input switches | SW0–SW15/BTNC drive assigned inputs | Not performed | No physical board | PENDING |
| HT-04 | No-error operation | status 00; corrected=encoded | Simulation passed | XSim waveform | PASS (RTL) |
| HT-05 | One-bit correction | status 01; original data/codeword recovered | Simulation passed | XSim waveform | PASS (RTL) |
| HT-06 | Two-bit correction | status 10; original data/codeword recovered | Simulation passed | XSim waveform | PASS (RTL) |
| HT-07 | Second two-bit position case | Correction succeeds | Simulation passed | XSim waveform | PASS (RTL) |
| HT-08 | Encoded output display | Encoded word visible on mapped outputs | Not performed | No physical board | PENDING |
| HT-09 | Corrected output display | Corrected word/data visible on mapped outputs | Not performed | No physical board | PENDING |
| HT-10 | Error status display | Status visible on mapped outputs | Not performed | No physical board | PENDING |
| HT-11 | Timing | 15 ns requirement met | WNS +1.286 ns; 0 failing endpoints | Vivado STA | PASS (Implementation) |
| HT-12 | Power | Physical power measurement | Not performed | 0.12 W is only a low-confidence Vivado estimate | PENDING |
| HT-13 | Long-duration operation | Stable long-duration hardware operation | Not performed | No physical board | PENDING |
| HT-14 | Physical acceptance | All board acceptance criteria met | Not performed | Hardware validation deferred | PENDING |
| HT-15 | Bitstream availability | Programming file generated | Completed | Vivado Write Bitstream Complete | PASS |
| HT-16 | DRC | No DRC errors | 0 errors | Vivado DRC | PASS |

The hardware test matrix records what can and cannot currently be accepted. Simulation/implementation evidence is PASS where available; physical tests are marked PENDING rather than fabricated.

# 12. Bring-Up Checklist

☐ Power verified — PENDING: no physical board

☐ FPGA programmed — PENDING: no physical board

☐ Clock verified — N/A for combinational RTL; virtual clock used only for STA

☐ Reset verified — N/A: no reset input in this combinational design

☐ Basic output verified — PENDING: no physical board

☒ Processor program loaded — N/A: project is not a processor

☐ Memory access verified — N/A: design contains no memory

☐ Pipeline behavior validated — N/A: design has no pipeline

☐ Long-duration test completed — PENDING: no physical board

# 13. Hardware Issues

| Issue ID | Symptom | Root Cause | Correction | Retest | Status |
| --- | --- | --- | --- | --- | --- |
| HW-01 | No Basys 3 board available | Physical board unavailable | Perform hardware validation when board is available | Not yet possible | Pending |
| HW-02 | No physical power measurement | Hardware test not performed | Measure board power during representative operation | Not yet possible | Pending |
| HW-03 | No physical I/O observation | Hardware test not performed | Verify LEDs, 7-seg, connectors and status outputs | Not yet possible | Pending |
| HW-04 | SAIF activity not available | Simulation activity logging did not match objects | Use only low-confidence Vivado estimate in current report | No retest required for implementation report | Documented |